NABTATECH SOLUTIONS

DOCUMENTATION SUITE

Nabtatech Network Configuration Report

| Organization | NabtaTech Solutions (Simulated Company) |
| --- | --- |
| Document Type | Formal Project Documentation |
| Version | 1.0 |
| Date | 2026-02-17 |

This document follows the unified NabtaTech documentation style guide with standardized cover layout, branding, header/footer structure, and formal section presentation.

# NabtaTech Solutions – Internal Network & File Server Deployment Report

## 1. Project Overview

This report documents the internal network infrastructure and the configuration of the Ubuntu File Server deployed for NabtaTech Solutions. The setup aims to provide a segmented, VLAN-based network supporting multiple departments, inter-VLAN routing, and centralized file services. The report includes network topology, switch and VLAN configurations, server deployment details, and verification steps to ensure full operational readiness.

## 2. Network Topology

Logical Network Layout

| Device/Segment | IP Range | VLAN | Purpose |
| --- | --- | --- | --- |
| HR Workstations | 192.168.10.0/24 | 10 | Human Resources department |
| Reception Workstations | 192.168.20.0/24 | 20 | Front desk and visitor management |
| Finance Workstations | 192.168.30.0/24 | 30 | Financial operations |
| IT Support Workstations | 192.168.40.0/24 | 40 | IT maintenance and support |
| DevOps Workstations | 192.168.50.0/24 | 50 | Development & operations |
| File Server (Ubuntu) | 192.168.100.10 | 100 | Centralized file storage |
| Web Server (Ubuntu) | 192.168.100.11 | 100 | Internal and external web services |
| Database Server (Ubuntu) | 192.168.100.12 | 100 | Centralized database services |

Connectivity Diagram:

## 3. Switch Configuration

Main Switch – VLAN and Trunk Setup

| Interface | Mode | VLAN Assigned/Allowed | Description |
| --- | --- | --- | --- |
| Fa0/1 | Trunk | 10,20,30,40,50,100 | Trunk to Core/Main Switch |
| Fa0/2 | Access | 20 | Reception |
| Fa0/3 | Access | 30 | Finance |
| Fa0/4-6 | Access | 10 | HR |
| Fa0/7-8 | Access | 40 | IT Support |
| Fa0/10 | Access | 50 | DevOps |
| Fa0/23 | Trunk | 10,20,30,40,50,100 | Trunk to File Server Switch |

Configuration Notes:

# Example trunk setup on main switch
interface fa0/1
 switchport mode trunk
 switchport trunk native vlan 100
 switchport trunk allowed vlan 10,20,30,40,50,100
 no shutdown

- Access ports are assigned to the corresponding VLAN for each department.

- Trunks carry multiple VLANs to the server switch and core router.

Spanning Tree & Connectivity Verification

- All VLANs are in forwarding state except VLANs not assigned locally.

- show spanning-tree vlan 100 confirms root bridge and port roles.

- CDP shows connectivity to other switches and router interfaces.

## 4. Ubuntu File Server Deployment

Server Specifications

| Parameter | Value |
| --- | --- |
| VM Name | File-Server |
| OS | Ubuntu 24.04 LTS |
| CPU | 2 cores |
| RAM | 4 GB |
| Storage | 40 GB VDI |
| Network Adapter | Bridged Adapter |
| VLAN | 100 |

Installation Steps

- Mount the Ubuntu Server ISO in VirtualBox under storage.

- Boot the VM and select Install Ubuntu Server.

- Configure language, location, and keyboard layout.

- Create an administrative user (mozan) and password.

- Choose default installation with standard utilities.

- Set hostname as File-Server.

- Configure network with static IP: 192.168.100.10/24, gateway: 192.168.100.1.

- Complete installation and reboot the VM.

Post-Installation Configuration

sudo apt update
sudo apt upgrade -y
sudo apt install build-essential dkms linux-headers-$(uname -r)

- Installed essential build tools and kernel headers for future services.

- Enabled network services for file sharing (NFS/Samba) in preparation for production use.

Network Verification

# Ping other VLAN gateways and servers
ping 192.168.10.1
ping 192.168.50.1
ping 192.168.100.11

- All pings successful, confirming inter-VLAN routing through the router.

## 5. Connectivity Tests

- Switch to Server: Verified trunk connectivity using show interfaces trunk.

- Ping Tests: Conducted from each VLAN to the File Server, all successful.

- Service Verification: SSH enabled and accessible from management workstation.

- Spanning Tree: All trunk links in forwarding state, no loops detected.

## 6. Troubleshooting Notes

- Native VLAN mismatches observed in CDP logs during initial testing; resolved by aligning trunk configurations on both switches.

- System services (systemd-resolved, systemd-networkd) initially failed during installation; reboot resolved service startup issues.

- Future installations should monitor ISO integrity and ensure proper VM network adapter type (bridged or host-only).

## 7. Next Steps

- Deploy Web Server and Database Server in VLAN 100.

- Configure NFS or Samba file sharing services on File Server.

- Perform additional connectivity and user access tests.

- Backup VM snapshots for all production servers.

Prepared by: Mozan Ahmed
Position: IT Network Engineer
Date: 15-Feb-2026